INDIAN AQUACULTURE GHG & CARBON MRV TOOL

v3.4 — IMC FIELD-DATA METHODOLOGY

Official Data Collection & Calculation Methodology Manual

For Farmers • Field Enumerators • MRV Practitioners • Third-Party Carbon Auditors

| Document Control | Specification |
| --- | --- |
| Tool | Indian Aquaculture GHG & Carbon MRV Tool v3.4 |
| Production system | Semi-intensive Indian Major Carp (IMC) polyculture |
| Pond system | Earthen pond; no mechanical aeration |
| Feed system | Plant-based / ingredient-blended feed with actual invoice-derived formulation |
| Accounting boundary | Scope 1, Scope 2 and relevant Scope 3 emissions |
| Primary use | Farm-level GHG screening, MRV preparation and carbon-reduction assessment |
| Methodology status | MRV-ready calculation framework; not a standalone carbon-credit standard or verification decision |

Critical scope rule: This manual applies only to the v3.4 IMC configuration. Mechanical aeration is not an allowable operating input for this configuration; Paddlewheel and Blower energy are therefore locked to zero.

Version 1.0  |  Methodology documentation aligned to workbook v3.4

# Document Structure

| Section | Content |
| --- | --- |
| Section 1 | MRV Data Collection Protocol (The Inputs) |
| 1.1 | Pre-Stocking / Baseline Enrollment |
| 1.2 | Daily Operational Logs |
| 1.3 | Weekly Water Quality & Plankton Log |
| 1.4 | Monthly / Mid-Cycle Checks |
| 1.5 | Harvest & Post-Harvest |
| 1.6 | Feed Formulation Inputs |
| Section 2 | The Calculation Engine (The Formulas) |
| 2.1 | Scope 3: Feed Manufacturing Emissions |
| 2.2 | Scope 1: Pond Biogeochemistry — The Boyd Engine |
| 2.3 | Scope 2: Energy Emissions |
| 2.4 | Dynamic Ecological Adjustments |
| 2.5 | Benthic Coupling — Soil Carbon |
| Section 3 | Carbon Credit Finalization & Integrity Checks |
| 3.1 | Net Emission & Carbon Reduction |
| 3.2 | MRV Integrity Traffic Lights |
| 3.3 | Exclusions & Caveats |

Use of this manual: Farm records are the primary evidence layer. The workbook is the calculation layer. Laboratory reports, invoices, meter readings, photographs, harvest records and other source documents are the audit evidence layer.

# Purpose, Scope and Methodological Position

The Indian Aquaculture GHG & Carbon MRV Tool v3.4 is a field-data-driven calculator designed for semi-intensive Indian Major Carp (IMC) polyculture in earthen ponds without mechanical aeration. The methodology replaces selected static defaults with measured or invoice-derived observations and uses bounded ecological adjustment factors for natural food availability, seasonal solar conditions, diurnal oxygen stress, plankton composition and sediment carbon dynamics.

The model preserves the biological population cascade used in the Growth & Feed Model and Harvest Schedule. Dynamic ecological variables are introduced primarily through feed requirement, pond biogeochemistry and emissions calculations rather than by rewriting the underlying population equations.

Carbon-credit boundary: A calculated emission reduction is not automatically a verified carbon credit. Any credit claim must satisfy the applicable carbon standard, methodology, baseline/additionality rules, monitoring requirements, uncertainty treatment, permanence requirements, validation/verification procedures and project-specific eligibility criteria.

# Section 1: MRV Data Collection Protocol (The Inputs)

All field data shall be traceable to a farm, pond, crop cycle and observation date. The minimum evidence chain is: source observation → recorded value → workbook input → calculated parameter → emissions result → audit trail.

## 1.1 Pre-Stocking / Baseline Enrollment

| Data category | Required input | Unit / format | Frequency | Evidence / verification |
| --- | --- | --- | --- | --- |
| Farm identity | Farm ID, pond ID, farmer/operator | Unique ID / text | Once per crop | Farm register; identity record |
| Geospatial | Latitude | Decimal degrees | Once per pond/crop | GPS/mobile coordinate record |
| Geospatial | Longitude | Decimal degrees | Once per pond/crop | GPS/mobile coordinate record |
| Pond geometry | Pond area | ha | Once per crop unless changed | Survey / GIS / farm record |
| Crop dates | Stocking date | Date | Once | Stocking register |
| Crop dates | Planned harvest date | Date | Once; update if changed | Farm plan |
| Soil baseline | Pre-stocking SOC | % | Pre-stocking | Laboratory report / sample ID |
| Soil baseline | Total soil nitrogen | % | Pre-stocking | Laboratory report |
| Soil baseline | Soil C:N ratio | ratio | Pre-stocking | Laboratory report or calculated result |
| Soil baseline | Bulk density | g/cm³ or t/m³ | Pre-stocking | Laboratory / field core measurement |
| Soil baseline | Sampling depth | m | Pre-stocking | Sampling sheet |
| Water baseline | pH | pH units | Baseline | Field meter / lab |
| Water baseline | Pre-dawn DO | mg/L | Baseline | Calibrated DO meter |
| Water baseline | TAN | mg/L | Baseline | Lab / validated field kit |
| Water baseline | H₂S | Nil / Detected | Baseline | Field/lab record |
| Stocking | Species composition | % / number | Stocking event | Stocking invoice/register |
| Stocking | Seed quantity | number | Stocking event | Stocking record |
| Stocking | Average seed weight | g/fish | Stocking event | Supplier/field record |
| Stocking | Initial survival assumption | % | Model input | Model assumption register |

## 1.2 Daily Operational Logs

| Input | Unit / allowed values | Minimum requirement | Primary evidence |
| --- | --- | --- | --- |
| Feed offered | kg/day | Record each feeding event or daily total | Feeding register / weighing record |
| Actual feed used | kg/day or kg/crop | Reconcile periodically | Feed register + stock balance |
| Pre-dawn DO | mg/L | Daily where feasible; critical days must be retained | Calibrated DO meter |
| H₂S | Nil / Detected | Daily observation where included in farm protocol | Field observation / test kit |
| Energy — grid | kWh | Meter/bill reading | Electricity meter/bill |
| Energy — diesel | L | Fuel consumed | Fuel log/invoice |
| Generator | kWh or L | Record use and fuel separately where possible | Generator/fuel log |
| Mortality | number / kg | Record daily mortality events | Mortality register |
| Water pumping | hours / kWh / diesel L | Record each pumping event | Pump log / meter / fuel record |
| Feeding time | clock time | Record actual feeding window | Feeding log |
| Punch Bag feeding | Yes / No | Record management status | Farm management log |

IMC aeration rule: Mechanical aeration is zero. Paddlewheel hours = 0 and Blower hours = 0. Any non-zero value is an MRV integrity error for an IMC crop and must be investigated before results are accepted.

## 1.3 Weekly Water Quality & Plankton Log

| Parameter | Unit / score | Why collected | Quality requirement |
| --- | --- | --- | --- |
| Date | date | Observation timestamp | Must be within crop period |
| pH | pH units | Water chemistry context | Calibrated meter |
| Pre-dawn DO | mg/L | Diurnal oxygen stress and anaerobic fraction | Pre-dawn measurement |
| TAN | mg/L | Nitrogen cycling context | Field kit/lab |
| H₂S | Nil / Detected | Anaerobic condition indicator | Consistent detection method |
| Secchi depth | cm | Water transparency / productivity proxy | Standard Secchi procedure |
| Diatoms | % | Positive plankton component | Microscopy/validated assessment |
| Green algae | % | Positive plankton component | Microscopy/validated assessment |
| Cyanobacteria | % | Risk modifier and N₂O adjustment trigger | Microscopy/validated assessment |
| Zooplankton score | 0–3 | Natural food availability indicator | 0 absent; 1 low; 2 moderate; 3 high |
| Data source | field/lab | Evidence classification | Required |
| Evidence ID | text | Audit traceability | Recommended |

Completeness rule: The v3.4 natural-feed credit is only activated when the weekly Water Quality & Plankton dataset is at least 75% complete for the expected weekly observation count. ≥90% is treated as high-quality MRV coverage.

## 1.4 Monthly / Mid-Cycle Checks

| Check | Required data | Purpose | Evidence |
| --- | --- | --- | --- |
| Biomass sampling | Sample weight, sample count, estimated standing biomass | Reconcile modeled and observed biomass | Sampling sheet/photos |
| Soil C:N | SOC%, total N%, C:N | Methanogenesis-risk adjustment | Laboratory report |
| Bulk density verification | g/cm³ | SOC mass calculation | Core sample record |
| Energy reconciliation | Grid kWh, diesel L, generator records | Close Scope 2 / fuel data gaps | Bills/invoices |
| Feed stock reconciliation | Opening stock, purchases, closing stock | Actual feed used | Inventory + invoices |
| Management practice check | Feeding window, Punch Bag status, C/N management | Scenario evidence | Farm log |

## 1.5 Harvest & Post-Harvest

| Input | Unit | Requirement | Evidence |
| --- | --- | --- | --- |
| Actual harvest weight | kg | Gross and/or net harvest quantity with clear basis | Harvest register / weighbridge |
| Species-wise harvest | kg | Preferred | Harvest register |
| Harvest date | date | Required | Harvest record |
| Actual feed used | kg | Final crop reconciliation | Feed inventory + invoices |
| Total feed invoices | kg and INR | All relevant crop purchases | Invoices |
| Post-harvest SOC | % | Same sampling framework as baseline | Laboratory report |
| Post-harvest soil C:N | ratio | Required where risk adjustment is used | Laboratory report |
| Post-harvest bulk density | g/cm³ | Required if changed from baseline | Laboratory/field record |
| Total energy | kWh / L | Final reconciliation | Bills/fuel records |
| Evidence archive | file/reference IDs | Link all major records | Digital audit folder |

## 1.6 Feed Formulation Inputs

| Ingredient | Required input | Unit | Evidence | Calculation use |
| --- | --- | --- | --- | --- |
| DOB | Actual purchased quantity | kg | Invoice | Actual mix %; blended Protein/C/N/EF |
| GNC | Actual purchased quantity | kg | Invoice | Actual mix %; blended Protein/C/N/EF |
| SBM | Actual purchased quantity | kg | Invoice | Actual mix %; blended Protein/C/N/EF |
| DDGS | Actual purchased quantity | kg | Invoice | Actual mix %; blended Protein/C/N/EF |
| Ingredient protein | Database value | % | Reference/database | Weighted protein |
| Ingredient carbon | Database value | % | Reference/database | Carbon mass balance |
| Ingredient nitrogen | Database value | % | Reference/database | Nitrogen mass balance |
| Ingredient EF | Database value | kgCO₂e/kg | Reference/database | Scope 3 feed emissions |
| Mix total | Σ actual kg | kg | Calculated | Must be >0 |

Invoice-first principle: The reference formulation is not the MRV formulation. For Scope 3 feed accounting, the v3.4 model uses actual ingredient quantities purchased and reconciled to crop use wherever data are available.

# Section 2: The Calculation Engine (The Formulas)

This section expresses the workbook calculation logic in mathematical form so that the same engine can be implemented in Excel, a database service or a web/mobile application. Variables must carry explicit units and provenance. Percent values are represented as fractions in equations unless otherwise stated.

## 2.1 Scope 3: Feed Manufacturing Emissions

For each ingredient i, the actual crop-period quantity is qᵢ (kg) and the ingredient emission factor is EFᵢ (kg CO₂e/kg). The blended feed emission factor is the mass-weighted average.

EF_blend = Σ(qᵢ × EFᵢ) / Σqᵢ

For IMC v3.4: EF_blend = (q_DOB·EF_DOB + q_GNC·EF_GNC + q_SBM·EF_SBM + q_DDGS·EF_DDGS) / (q_DOB + q_GNC + q_SBM + q_DDGS)

Feed CO₂e = Feed_used × EF_blend

Where Feed_used is the reconciled crop-period feed quantity used for MRV. If only purchases are available, the workbook may use purchases as a temporary proxy, but opening and closing inventory must be reconciled before a final credit claim.

### Weighted nutrient profile

Protein_blend = Σ(qᵢ × Proteinᵢ) / Σqᵢ

Carbon_blend = Σ(qᵢ × Carbonᵢ) / Σqᵢ

Nitrogen_blend = Σ(qᵢ × Nitrogenᵢ) / Σqᵢ

The actual mix percentages are pᵢ = qᵢ / Σqᵢ and must satisfy Σpᵢ = 1.00, subject to a numerical tolerance defined in the workbook.

## 2.2 Scope 1: Pond Biogeochemistry — The Boyd Engine

The pond engine follows a mass-balance framing in which feed-derived carbon and nitrogen are partitioned among retained biomass and environmental loss pathways. The workbook uses these balances as the bridge between production inputs and pond GHG emissions.

### Carbon mass balance

C_feed = Feed_mass × C_fraction

C_retained = Biomass_gain × C_fraction_of_fish_biomass

C_lost_to_pond = C_feed − C_retained

C_feed ≈ C_retained + C_lost_to_pond

MRV tolerance: The carbon balance should be treated as passing only when the absolute residual is within the workbook's defined tolerance. A material residual indicates an input, unit or accounting-boundary error.

### Nitrogen mass balance

N_feed = Feed_mass × N_fraction

N_retained = Biomass_gain × N_fraction_of_fish_biomass

N_lost_to_pond = N_feed − N_retained

N_feed ≈ N_retained + N_lost_to_pond

### Dynamic Anaerobic Fraction

The anaerobic fraction is no longer a purely static 20% input. It is derived from the weekly field log using average pre-dawn dissolved oxygen and H₂S detection.

AF = 12%  if  (DO_pre-dawn,avg ≥ 4.0 mg/L) AND (H₂S = Nil)

AF = 30%  if  (DO_pre-dawn,avg < 3.0 mg/L) AND (H₂S = Detected)

AF = 20%  otherwise

If no usable water-quality observations are available, the model retains the controlled fallback default of 20%, and the parameter must be marked as DEFAULT rather than MEASURED.

### Soil C:N Adjustment Factor

C:N_adj = 1.00  if C:N ≤ 20

C:N_adj = 1.15  if 20 < C:N ≤ 30

C:N_adj = 1.25  if C:N > 30

The factor is a bounded risk adjustment representing elevated anaerobic/methanogenic potential at high sediment C:N. It is not a direct measurement of methanogenesis.

### Methane (CH₄) emission formula

The v3.4 implementation expresses methane generation as the relevant anaerobic carbon pool multiplied by the soil C:N adjustment, followed by sediment carbon burial and methane oxidation terms.

C_anaerobic = C_lost_to_pond × AF × C:N_adj

C_available_for_CH4 = C_anaerobic × (1 − f_burial)

CH₄_C = C_available_for_CH4 × f_CH4-C

CH₄_emitted = CH₄_C × (1 − f_oxidation) × (16/12)

Where f_burial is the sediment carbon burial fraction, f_CH4-C is the fraction of anaerobically processed carbon represented as methane carbon in the selected model parameterization, and f_oxidation is the methane oxidation fraction. The exact numerical defaults and source references remain controlled by the Scientific References sheet.

### Nitrous oxide (N₂O) emission formula

EF_N2O,adj = EF_N2O × 1.20  if Cyanobacteria_avg > 50%

EF_N2O,adj = EF_N2O  otherwise

N₂O-N = N_relevant × EF_N2O,adj

N₂O = N₂O-N × (44/28)

The +20% cyanobacteria adjustment is a model coefficient and must be treated as a controlled assumption. It is triggered only when the average cyanobacteria percentage exceeds 50% under the v3.4 rule.

## 2.3 Scope 2: Energy Emissions

For the IMC configuration, mechanical aeration is excluded. Energy inputs are restricted to water pumping and generator/electricity consumption supported by farm records.

Grid CO₂e = Electricity_used (kWh) × CEA_grid_EF (kg CO₂e/kWh) ÷ 1000

Diesel CO₂e = Diesel_used (L) × Diesel_EF (kg CO₂e/L) ÷ 1000

Energy CO₂e = Grid CO₂e + Diesel CO₂e + other eligible energy sources

The CEA grid emission factor must be stored with the applicable database version, financial/assessment year and source reference. The workbook must never silently substitute an unrecorded grid EF version.

IMC energy lock: Paddlewheel energy = 0 kWh and Blower energy = 0 kWh. A non-zero value creates an MRV FAIL condition for the IMC configuration.

## 2.4 Dynamic Ecological Adjustments (v3.4 Specifics)

### Seasonal Photosynthesis Factor (SPF)

SPF is a bounded 0–1 proxy for seasonal natural-food potential. It uses latitude, crop dates, day length and extraterrestrial solar radiation. The factor is not a direct measurement of pond photosynthesis; it is a seasonal forcing variable that is subsequently moderated by measured plankton and water-quality data.

δ = 0.409 × sin(2πJ/365 − 1.39)

ω_s = arccos[−tan(φ) × tan(δ)]

N = (24/π) × ω_s

R_a = (24×60/π) × 0.0820 × d_r × [ω_s sinφ sinδ + cosφ cosδ sinω_s]

SPF = clamp(0, 1, 0.80 × SolarRadiationFactor + 0.20 × DaylightFactor)

φ is latitude in radians; J is day of year; d_r = 1 + 0.033 cos(2πJ/365). The v3.4 implementation uses solar radiation as the dominant component and daylight as a secondary component.

### Plankton Quality Index (PQI)

AQI = min[1, (Diatoms% + GreenAlgae%) / 60%]

ZQI = ZooplanktonScore / 3

PQI = 0.60 × AQI + 0.40 × ZQI

Cyanobacteria are not treated as a positive PQI component. They are handled as a separate risk modifier.

### Natural Feed Offset

NaturalOffset = 0, if WQ completeness < 75%

NaturalOffset = min(20%, 20% × PQI × SPF), if Cyanobacteria_avg ≤ 50%

NaturalOffset = min(5%, 20% × PQI × SPF), if Cyanobacteria_avg > 50%

AdjustedFeedRequirement = BaselineFeedRequirement × (1 − NaturalOffset) × FCR/DO adjustments

Interpretation: NaturalOffset is a bounded modelled credit for natural food availability. It must not be presented as direct measurement of avoided feed. Actual feed used remains the preferred MRV quantity for emissions accounting.

### Punch Bag Feeding System

FCR_PunchBag = FCR_baseline × (1 − 0.125)  when Punch Bag = Yes

FCR_PunchBag = FCR_baseline  when Punch Bag = No

The 12.5% value is the midpoint of the v3.4 10–15% indicative improvement range. The separate 10% wastage-reduction KPI is retained as a management indicator and is not multiplied again into the FCR calculation, preventing double counting.

### Diurnal DO Stress Penalty

DO_stress_factor = 1.10, if average Pre-dawn DO < 3.0 mg/L

DO_stress_factor = 1.00, otherwise

FCR_effective = FCR_adjusted × DO_stress_factor

The 10% penalty is a controlled model coefficient. It represents increased stress-related inefficiency and environmental loading under repeated low pre-dawn oxygen conditions.

## 2.5 Benthic Coupling — Soil Carbon

The v3.4 soil carbon calculation converts the change in measured soil organic carbon concentration into an estimated mass of carbon in the sampled soil layer.

ΔSOC_mass (t C) = [(SOC_post − SOC_pre) / 100] × BD × Depth × Area_ha × 10,000

SOC_pre and SOC_post are percentages; BD is bulk density in t/m³ (numerically equal to g/cm³); Depth is sampling depth in metres; Area_ha is pond area in hectares; 10,000 converts hectares to square metres.

ΔSOC_annualized = ΔSOC_mass × 365 / (PostTestDate − PreTestDate)

CO₂e_equivalent = ΔSOC_annualized × (44/12)

Sign convention: Positive ΔSOC indicates an increase in measured sediment carbon; negative ΔSOC indicates depletion. The signed result must be retained in the audit record.

# Section 3: Carbon Credit Finalization & Integrity Checks

## 3.1 Net Emission & Carbon Reduction

The carbon-reduction comparison is made between a defined baseline scenario and the improved-farming scenario using the same production boundary, functional unit, crop duration and emission categories.

Baseline_Net_Emission = Scope1_baseline + Scope2_baseline + Scope3_baseline

Improved_Net_Emission = Scope1_improved + Scope2_improved + Scope3_improved

Gross_Emission_Reduction = Baseline_Net_Emission − Improved_Net_Emission

The final report shall state whether the result is a screening estimate, an internal MRV result or a creditable reduction under a specified external methodology. No carbon-credit claim shall be generated solely from the spreadsheet result.

## 3.2 MRV Integrity Traffic Lights

The v3.4 engine should classify each control as PASS, REVIEW or FAIL. A final credit-claim dataset should not be released when a critical control is FAIL.

| Control | PASS condition | REVIEW condition | FAIL condition |
| --- | --- | --- | --- |
| IMC Aeration Lock | Paddlewheel = 0 and Blower = 0 | Not applicable | Any non-zero mechanical aeration input |
| Crop dates | Valid stocking and harvest dates | Date changed after plan | Missing or harvest ≤ stocking |
| Feed mix | Actual ingredient mix sums to 100% | Temporary reference mix only | Mix total ≠100% or zero total |
| Feed evidence | Invoices + inventory reconciliation | Invoices incomplete | No traceable feed quantity for final MRV |
| WQ completeness | ≥90% | 75–89% | <75% when ecological credit is claimed |
| Cyanobacteria | 0–50% with evidence | >50% with evidence | Invalid / missing values used as measured |
| DO integrity | Valid pre-dawn observations | Partial coverage | Invalid units / impossible values |
| Harvest vs model | Within defined tolerance | Variance requires review | Exceeds theoretical maximum materially |
| Carbon balance | Residual within tolerance | Small unexplained residual | Material imbalance |
| Nitrogen balance | Residual within tolerance | Small unexplained residual | Material imbalance |
| SOC test integrity | Pre + post tests, matched depth/area basis | One parameter estimated | Missing/invalid baseline or post test when SOC result is reported |
| Soil C:N | Traceable laboratory result | Older but applicable result | Unverified value used for adjustment |
| Energy EF | CEA version/year recorded | Reference pending | Uncontrolled EF |
| Actual energy | Bills/meter/fuel records reconciled | Partial records | Untraceable energy quantity |
| Scenario integrity | Improvement actions evidenced | Partial evidence | Claimed action not implemented |

### Automated integrity formulas

MRV_status = FAIL if any critical control = FAIL; else REVIEW if any control = REVIEW; else PASS

WQ_completeness = min(1, ObservedWeeklyRecords / ExpectedWeeklyRecords)

ExpectedWeeklyRecords = ROUNDUP(CropDays / 7, 0)

HarvestCheck = PASS if ActualHarvest ≤ TheoreticalMaximumBiomass × (1 + tolerance)

FeedMixCheck = PASS if |ΣMixPercent − 1| ≤ tolerance

SOCCheck = PASS if exactly one valid Pre-Stocking and one valid Post-Harvest observation exist for the reported SOC result

## 3.3 Exclusions & Caveats

| Item | v3.4 treatment | Reason |
| --- | --- | --- |
| Sediment carbon sequestration | Informational; not netted into final credit claim | Permanence, reversibility, sampling uncertainty and project-boundary requirements require separate treatment |
| Better Aeration | Disabled for IMC | Mechanical aeration is outside the defined semi-intensive IMC configuration |
| Natural feed offset | Bounded model adjustment | Plankton observations support potential natural-food contribution but do not directly measure avoided artificial feed |
| SPF | Seasonal proxy | Solar/daylight forcing is not a direct measurement of pond primary production |
| Cyanobacteria +20% N₂O factor | Controlled model coefficient | Must be treated as a documented adjustment assumption, not a universal emission factor |
| DO stress +10% FCR | Controlled model coefficient | Represents a bounded stress adjustment; requires consistent field DO data |
| C:N methanogenesis adjustment | Risk modifier | High C:N indicates elevated risk but does not directly measure CH₄ production |
| Carbon-credit claim | Requires external methodology and verification | Spreadsheet calculations alone do not establish additionality, baseline eligibility, permanence or verification |

Auditor-facing rule: Every material calculated parameter should be traceable to either MEASURED data, DERIVED data from measured inputs, or a controlled DEFAULT. The provenance class must be retained with the result.

# Data Provenance and Evidence Classification

| Status | Definition | Examples | Use in final MRV |
| --- | --- | --- | --- |
| MEASURED | Directly observed or laboratory/instrument result | Pre-dawn DO, SOC%, kWh meter, harvest weight | Preferred |
| INVOICED | Supported by commercial record | Feed ingredient kg, diesel litres | Preferred where applicable |
| DERIVED | Calculated only from measured/invoiced inputs | Blended feed EF, PQI, anaerobic fraction | Allowed with formula traceability |
| DEFAULT | Controlled literature/model assumption | Fallback EF, bounded coefficients | Must be disclosed |
| ESTIMATED | Modelled proxy not directly observed | SPF, natural-feed offset | Allowed only within stated bounds and caveats |

# Field Enumerator Quick Checklist

Confirm Farm ID, Pond ID and crop dates before collecting measurements.

Record latitude and longitude once per pond/crop cycle and retain the GPS source.

Verify that the IMC configuration has zero Paddlewheel and zero Blower hours.

Record feed quantity by weighing whenever possible; do not rely only on visual estimates.

Retain all feed purchase invoices for DOB, GNC, SBM and DDGS.

Take pre-dawn DO measurements consistently at the defined monitoring point.

Record H₂S as Nil or Detected using a consistent method.

Complete the weekly plankton log even when the pond appears visually normal.

Record cyanobacteria percentage separately; do not combine it with beneficial algal groups.

Record zooplankton using the defined 0–3 score.

Collect pre-stocking and post-harvest soil samples using comparable depth and sampling protocol.

Reconcile feed inventory, feed purchases and feed used before finalizing the crop.

Reconcile electricity bills, meter readings, diesel records and generator use.

Attach evidence IDs to major observations and calculations.

Resolve all MRV FAIL conditions before issuing a final carbon-reduction result.

# Software Implementation Notes

The formulas in this manual are intentionally expressed independently of Excel cell addresses. A web implementation should represent the same variables as typed fields with units, validation rules, timestamps and provenance. The calculation engine should retain intermediate values so an auditor can reproduce the result without reverse-engineering a single final number.

| Data object | Minimum fields |
| --- | --- |
| Farm | farm_id, operator, latitude, longitude, pond_area_ha |
| Crop | crop_id, farm_id, stocking_date, harvest_date, species_configuration |
| FeedPurchase | date, ingredient, quantity_kg, invoice_id, supplier |
| FeedUse | date, quantity_kg, feeding_event_id |
| WaterQualityObservation | date, pH, pre_dawn_DO, TAN, H2S, secchi_depth |
| PlanktonObservation | date, diatoms_pct, green_algae_pct, cyanobacteria_pct, zooplankton_score |
| SoilTest | date, phase, SOC_pct, total_N_pct, C_N, bulk_density, depth_m, lab_report_id |
| EnergyRecord | date, source, quantity, unit, meter_or_invoice_id |
| Harvest | date, species, quantity_kg, evidence_id |
| CalculatedParameter | parameter, value, unit, provenance, formula_version, timestamp |
| MRVCheck | check_id, status, observed_value, threshold, evidence_id |

# Methodology Version Control

| Version | Change |
| --- | --- |
| v3.4 | Field-data-driven IMC MRV architecture: SPF, WQ/plankton log, dynamic anaerobic fraction, cyanobacteria N₂O adjustment, DO stress penalty, actual feed formulation, measured SOC, C:N adjustment, IMC aeration lock and MRV integrity checks. |
| Future revisions | Any change to emission factors, adjustment coefficients, boundaries, units, equations or default assumptions must be recorded in the workbook Change Log and this manual before use in a new reporting period. |

# Final Methodology Statement

The Indian Aquaculture GHG & Carbon MRV Tool v3.4 is designed to convert farm observations into a transparent, reproducible and auditable emissions dataset for semi-intensive IMC polyculture. Its principal methodological improvement is the controlled transition from static defaults toward field-data-driven parameters while preserving the established production and harvest cascade.

For carbon-credit applications, the workbook result shall be treated as a quantified MRV output subject to the requirements of the selected external carbon standard and methodology. The final claim shall use only eligible emission reductions supported by the defined project boundary, baseline, monitoring plan, evidence archive, uncertainty treatment and independent validation/verification process.